Safety Data Sheet

Material Name: SURFACEPREP CONCENTRATE

# Section 1 - PRODUCT AND COMPANY IDENTIFICATION

Material Name

SURFACEPREP CONCENTRATE

## Product Code

10-P-237-55

Use:   Cleaner and Degreaser Concentrate.

Description:  Translucent chartreuse liquid

Manufacturer:

Aegis Sealants, LLC

4893 Klatte Road

Cincinnati, Ohio 4524

Product Information:  513-932-3396

Emergency Phone #: 513.315.7150

Issue Date July 1, 2025

# Section 2 - HAZARDS IDENTIFICATION

Classification in accordance with paragraph (d) of 29 CFR 1910.1200.

Skin Corrosion/Irritation - Category 2

Serious Eye Damage/Eye Irritation - Category 2A

Skin Sensitization – Category 1

Specific target organ toxicity - Single exposure - Category 3

## GHS Label Elements  Symbol(s)

Signal Word

Warning

## Hazard Statement(s)

Causes skin and serious eye irritation.

May cause an allergic skin reaction. May cause respiratory irritation.

## Precautionary Statement(s)  Prevention

Wash thoroughly after handling. Wear protective gloves and eye/face protection. Avoid breathing dust/fume/gas/mist/vapors/spray. Use only outdoors or in a well-ventilated area.

## Response

IF ON SKIN (or hair): Remove/take off immediately all contaminated clothing. Rinse skin with water/shower. Wash contaminated clothing before reuse. If skin irritation or rash occurs:  Get medical advice/attention.  IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. IF INHALED: Remove person to fresh air and keep comfortable for breathing. Call a POISON CENTER or doctor/physician if you feel unwell.  IF SWALLOWED: Immediately call a POISON CENTER or doctor/physician.

## Storage

Store in a well-ventilated place. Keep container tightly closed.  Store locked up. Disposal

Dispose of contents/container in accordance with local/regional/national/international regulations.

Other Hazards

No additional information is available.

# Section 3 - COMPOSITION / INFORMATION ON INGREDIENTS

| CAS | Component Name | Percent |
| --- | --- | --- |
| 7732-18-5 | Water | 75-100 |
| 68439-57-6 | Sodium alkyl olefin sulfonate | 1-4 |
| 34590-94-8 | Dipropylene glycol monomethyl ether | 1-5 |
| 1569-01-3 | 2-Propanol, 1-propoxy- | 1-3 |
| 34398-01-1 | Poly(oxy-1,2-ethanediyl), .alpha.-undecyl-.omega.hydroxy- | 1-3 |
| 6834-92-0 | Silicic acid, disodium salt | 1-2 |
| PRODUCT DILUTION FOR USE RANGES are as follows: LIGHT: 1 to 50 with water MEDIUM: 1 to 20 with water HEAVY: 1 to 4 with water Actual WT% of product constituents will vary according to dilution. |  |  |

# Section 4 - FIRST AID MEASURES

## Inhalation

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing. Call a POISON CENTER or doctor/physician if you feel unwell.

Skin

IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water/shower. If skin irritation occurs:  Get medical advice/attention. Wash contaminated clothing before reuse.

Eyes

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.  If eye irritation persists:  Get medical advice/attention. Ingestion

IF SWALLOWED: Rinse mouth. Do NOT induce vomiting. Get medical attention. Most

Important Symptoms/Effects

## Acute

Causes eye and skin burns. Respiratory tract irritation.  Allergic skin reaction. Delayed

No information on significant adverse effects.

## Indication of any immediate medical attention and special treatment needed

Treat symptomatically and supportively. Treatment may vary with condition of victim and specifics of incident. Call 1-800-468-1760 for additional information.

# Section 5 - FIRE FIGHTING MEASURES

## Extinguishing Media  Suitable Extinguishing Media

Carbon dioxide, alcohol-resistant foam, dry chemical, water spray, or water fog.

Unsuitable Extinguishing Media None known.

Special Hazards Arising from the Chemical  Product may burn, but does not ignite readily.

## Hazardous Combustion Products

Decomposition and combustion materials may be toxic. Burning may produce carbon monoxide and unidentified organic compounds.

Fire Fighting Measures

Keep storage containers cool with water spray.

## Special Protective Equipment and Precautions for Firefighters

A positive-pressure, self-contained breathing apparatus (SCBA) and full-body protective equipment are required for fire emergencies.

# Section 6 - ACCIDENTAL RELEASE MEASURES

Personal Precautions, Protective Equipment and Emergency Procedures

Wear personal protective clothing and equipment, see Section 8. Avoid release to the environment.

## Methods and Materials for Containment and Cleaning Up

Remove all ignition sources. Do not touch or walk through spilled product. Stop leak if you can do it without risk. Isolate hazard area. Keep unnecessary and unprotected personnel from entering. Ventilate area and avoid breathing vapor or mist. A vapor suppressing foam may be used to reduce vapors. Contain spill away from surface water and sewers. Sorb with compatible sorbent material and shovel with a clean tool into a sealable container for disposal. Additionally, for large spills: Dike far ahead of liquid spill for collection and later disposal.

# Section 7 - HANDLING AND STORAGE

## Precautions for Safe Handling

Keep away from heat, sparks, or flame. Where flammable mixtures may be present, equipment safe for such locations should be used. Use clean tools. When transferring product, metal containers, including trucks and tank cars, should be grounded and bonded. Avoid breathing vapor or mist. Use only outdoors or in a well ventilated area. Avoid contact with eyes, skin, clothing, shoes. Do not eat, drink, or smoke when using this product. Wear suitable protective gloves and eye/face protection. Wash thoroughly after handling.

| MIX RATIO | g/L VOC | VOC CONCENTRATION (WT %) |
| --- | --- | --- |
| LIGHT: 1 part concentrate: 50 parts water | 1.6 | 0.15 |
| MEDIUM: 1 part concentrate: 20 parts water | 3.8 | 0.37 |
| HEAVY (FOR CALIFORNIA): 1part concentrate: 15 parts water | 5.0 | 0.48 |
| HEAVY: 1 part concentrate: 4 parts water (NOT FOR USE IN CALIFORNIA) | 15.8 | 1.54 |

## Conditions for Safe Storage, Including any Incompatibilities

Store in a well-ventilated place.  Keep container tightly closed when not in use and during transport. Store containers in a cool, dry, well-ventilated place. Keep containers away from heat, flame, sparks, static electricity, or other sources of ignition. Empty product containers may retain product residue and can be dangerous. Store between 40-90°F (4°C-32°C). Prevent product from freezing. Store locked up.

Incompatible Materials

Avoid strong acids, oxidizing materials, combustible materials.

# Section 8 - EXPOSURE CONTROLS / PERSONAL PROTECTION

## Component Exposure Limits

| Dipropylene glycol monomethyl ether | 34590-94-8 |
| --- | --- |
| ACGIH: | 100 ppm TWA; 150 ppm STEL; Skin - potential significant contribution to overall exposure by the cutaneous route |
| NIOSH: | 100 ppm TWA ; 600 mg/m3 TWA; 150 ppm STEL ; 900 mg/m3 STEL Potential for dermal absorption; 600 ppm IDLH |
| OSHA (US): | 100 ppm TWA ; 600 mg/m3 TWA; prevent or reduce skin absorption |
| Alberta | 100 ppm TWA ; 606 mg/m3 TWA ; 150 ppm STEL ; 909 mg/m3 STEL ; Substance may be readily absorbed through intact skin |
| British Columbia; Northwest Territories; Nunavut | 100 ppm TWA; Skin notation; 150 ppm STEL |
| Manitoba | 100 ppm TWA; Skin - potential for cutaneous absorption Skin - potential significant contribution to overall exposure by the cutaneous route |
| New Brunswick | 100 ppm TWA ; 606 mg/m3 TWA; 150 ppm STEL ; 909 mg/m3 STEL ; Skin - potential for cutaneous absorption |
| Nova Scotia | 100 ppm TWA; 150 ppm STEL; Skin - potential significant contribution to overall exposure by the cutaneous route |
| Ontario | 100 ppm TWA; 150 ppm STEL; Danger of cutaneous absorption |
| Prince Edward Island | 100 ppm TWA; 150 ppm STEL |
| Quebec | 100 ppm TWAEV ; 606 mg/m3 TWAEV; 150 ppm STEV ; 909 mg/m3 STEV; Skin designation |
| Saskatchewan | 100 ppm TWA; 150 ppm STEL ; Potentially harmful after absorption through skin or mucous membranes |

ACGIH - Threshold Limit Values - Biological Exposure Indices (BEI)  There are no biological limit values for any of this product's components.

## Engineering Controls

Provide general ventilation needed to maintain concentration of vapor or mist below applicable exposure limits. Where adequate general ventilation is unavailable, use process enclosures, local exhaust ventilation, or other engineering controls to control airborne levels below applicable exposure limits.

## Individual Protection Measures, such as Personal Protective Equipment  Eye/face protection

Wear safety glasses.  Additional protection like goggles, face shields, or respirators may be needed dependent upon anticipated use and concentrations of mists or vapors.  Eye wash fountain and emergency showers are recommended.  Contact lens use is not recommended.

## Respiratory Protection

A respiratory protection program which meets USA's OSHA General Industry Standard 29 CFR 1910.134 or Canada's CSA Standard Z94.4-M1982 requirements must be followed whenever workplace conditions warrant a respirator's use. Consult a qualified Industrial Hygienist or Safety Professional for respirator selection guidance.

## Skin Protection/Glove Recommendations

Wear chemical resistant protective gloves; use of natural rubber or equivalent gloves is not recommended. Where spills are likely, wear appropriate chemical-resistant apron, coveralls, long sleeve shirts, or other protective clothing.

## Protective Materials

Personal protective equipment should be selected based upon the conditions under which this material is used. A hazard assessment of the work area for PPE requirements should be conducted by a qualified professional pursuant to regulatory requirements. The following PPE should be considered the minimum required: Safety glasses, Gloves, and Lab coat or apron.

# Section 9 - PHYSICAL AND CHEMICAL PROPERTIES

| Appearance | Yellow liquid | Physical State | Liquid |
| --- | --- | --- | --- |
| Odor | Sassafras | Color | Yellow |
| Odor Threshold | Not available | pH | 12.7 (undiluted ) |
| Melting Point | Not available | Boiling Point | >93 °C (200 °F ) |
| Boiling Point Range | Not available | Freezing point | Not available |
| Evaporation Rate | 1 (Butyl acetate = 1 ) | Flammability (solid, gas) | Not available |
| Autoignition Temperature Not available |  | Flash Point | >93.3 °C (200 °F ) |
| Lower Explosive Limit | Not available | Decomposition temperature Not available |  |
| Upper Explosive Limit | Not available | Vapor Pressure Not available |  |
| Vapor Density (air=1) | Not available | Specific Gravity (water=1) 1.029 (Water = 1 ) |  |
| Water Solubility | (Complete ) | Partition coefficient: n-octanol/water Not available |  |
| Viscosity | Not available | Kinematic viscosity Not available |  |
| Solubility (Other) | Not available | 8.52 - 8.62 lb/gal Density (US ) |  |
| Molecular Weight | Not available |  |  |
| Volatile Organic Compounds (As Regulated) | 7.7% or less, depending on use dilution. As per U.S EPA 40 CFR 51.100(s) See Section 7, Handling and Storage, for VOC of diluted product. Consult your state or local air district regulations for location specific information. |  |  |

# Section 10 - STABILITY AND REACTIVITY

Reactivity

No reactivity hazard is expected.

Chemical Stability

Stable under normal temperatures and pressures.

Possibility of Hazardous Reactions

Polymerization is not known to occur under normal temperature and pressures. Not reactive with water.

Conditions to Avoid

Avoid excessive heat and ignition sources.

Incompatible Materials

Avoid strong acids, oxidizing materials, combustible materials.

## Hazardous decomposition products

Decomposition and combustion materials may be toxic. Burning may produce carbon monoxide and unidentified organic compounds.

# Section 11 - TOXICOLOGICAL INFORMATION

## Information on Likely Routes of Exposure  Inhalation

May cause respiratory tract irritation. May cause headache, drowsiness, dizziness, loss of coordination, liver effects, difficulty breathing, cough, chest pain, lung congestion.

Skin Contact

Causes skin burns.

## Eye Contact

Causes eye burns.  Ingestion

May cause loss of coordination, headache, drowsiness, dizziness, nausea, vomiting.

Acute and Chronic Toxicity  Component Analysis - LD50/LC50

The components of this material have been reviewed in various sources and the following selected endpoints are published:

Water (7732-18-5)

Oral LD50 Rat >90 mL/kg

Sodium alkyl olefin sulfonate (68439-57-6)

Oral LD50 Rat 2220 mg/kg; Dermal LD50 Rabbit >740 mg/kg

Dipropylene glycol monomethyl ether (34590-94-8)

Oral LD50 Rat 5.35 g/kg; Dermal LD50 Rabbit 9500 mg/kg

2-Propanol, 1-propoxy- (1569-01-3)

Oral LD50 Rat 2490 mg/kg; Dermal LD50 Rabbit 3550 mg/kg

Silicic acid, disodium salt (6834-92-0)

Oral LD50 Rat 1153 mg/kg

## Product Toxicity Data

Acute Toxicity Estimate  No data available.

## Immediate Effects  Causes burns.

## Delayed Effects  No information on significant adverse effects.

## Irritation/Corrosivity Data

Causes severe skin burns and eye damage.  Causes severe damage to the respiratory tract. Respiratory Sensitization

Based on best current information, there is no known human sensitization associated with this product.

Dermal Sensitization

Based on best current information, there is no known human sensitization associated with this product.

Component Carcinogenicity

None of this product's components are listed by ACGIH, IARC, NTP, DFG or OSHA.

Germ Cell Mutagenicity

Based on best current information, there is no known mutagenicity associated with this product.

Tumorigenic Data

Based on best current information, there is no known teratogenicity associated with this product.

## Reproductive Toxicity

Based on best current information, there is no known reproductive toxicity associated with this product. Also see SECTION 15: CALIFORNIA.

Specific Target Organ Toxicity - Single Exposure  No target organs identified.

Specific Target Organ Toxicity - Repeated Exposure  No target organs identified.

Aspiration hazard

No information available for the product.

## Medical Conditions Aggravated by Exposure

Individuals with pre-existing respiratory tract (nose, throat, and lungs), central nervous system, kidney, eye, and/or skin disorders may have increased susceptibility to the effects of exposure.

Additional Data

No additional information is available.

# Section 12 - ECOLOGICAL INFORMATION

## Component Analysis - Aquatic Toxicity

| Sodium alkyl olefin sulfonate | 68439-57-6 |
| --- | --- |
| Fish: | LC50 96 h Brachydanio rerio 1 - 10 mg/L [static ]; LC50 96 h Brachydanio rerio 12.2 mg/L [semi-static ] |
| Dipropylene glycol monomethyl ether | 34590-94-8 |
| Fish: | LC50 96 h Pimephales promelas >10000 mg/L [static ] |
| Invertebrate: | LC50 48 h Daphnia magna 1919 mg/L IUCLID |
| 2-Propanol, 1-propoxy- | 1569-01-3 |
| Fish: | LC50 96 h Oncorhynchus mykiss >100 mg/L [static ] |
| Silicic acid, disodium salt | 6834-92-0 |
| Fish: | LC50 96 h Brachydanio rerio 210 mg/L [semi-static ]; LC50 96 h Brachydanio rerio 210 mg/L |

Persistence and Degradability

No information available for the product.

## Bioaccumulative Potential

No information available for the product.  Mobility

No information available for the product.

Other Toxicity

No additional information is available.

# Section 13 - DISPOSAL CONSIDERATIONS

## Disposal Methods

Dispose in accordance with federal, state, provincial, and local regulations. Regulations may also apply to empty containers. The responsibility for proper waste disposal lies with the owner of the waste. Contact Safety-Kleen regarding proper recycling or disposal.

# Section 14 - TRANSPORT INFORMATION

US DOT Information:   Not regulated for transport.

IATA Information:   Not regulated for transport.

IMDG Information:  Not regulated for transport.

TDG Information:   Not regulated for transport.

International Bulk Chemical Code

This material does not contain any chemicals required by the IBC Code to be identified as dangerous chemicals in bulk.

# Section 15 - REGULATORY INFORMATION

## Canada Regulations  CEPA - Priority Substances List

None of this product's components are on the list.  Ozone Depleting Substances

None of this product's components are on the list.

Council of Ministers of the Environment - Soil Quality Guidelines  None of this product's components are on the list.

## Council of Ministers of the Environment - Water Quality Guidelines  None of this product's components are on the list.  U.S. Federal Regulations

None of this product's components are listed under SARA Sections 302/304 (40 CFR 355 Appendix A), SARA Section 313 (40 CFR 372.65), CERCLA (40 CFR 302.4), TSCA 12(b), or require an OSHA process safety plan.

## SARA Section 311/312 (40 CFR 370 Subparts B and C) reporting categories

Skin Corrosion/Irritation; Serious Eye Damage/Eye Irritation; Specific Target Organ Toxicity  U.S. State Regulations

The following components appear on one or more of the following state hazardous substances lists:

| Component | CAS | CA | MA | MN | NJ | PA |
| --- | --- | --- | --- | --- | --- | --- |
| Dipropylene glycol monomethyl ether | 34590-94-8 | Yes | Yes | Yes | Yes | Yes |

California Safe Drinking Water and Toxic Enforcement Act (Proposition 65)  Not listed under California Proposition 65.

## Component Analysis - Inventory  Water (7732-18-5)

| US | CA | AU | CN |  | EU |  | JP - ENCS | JP - ISHL |  | KR KECI - Annex 1 | KR KECI - Annex 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Yes | DSL | Yes | Yes |  | EIN |  | Yes | No |  | Yes | No |
| KR - REACH CCA |  |  |  | MX |  | NZ | PH | TH- TECI | TW | VN (Draft) |  |
| No |  |  |  | Yes |  | Yes | Yes | Yes | Yes | Yes |  |

## Sodium alkyl olefin sulfonate (68439-57-6)

| US | CA | AU | CN |  | EU |  | JP - ENCS | JP - ISHL |  | KR KECI - Annex 1 | KR KECI - Annex 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Yes | DSL | Yes | Yes |  | EIN |  | No | No |  | Yes | No |
| KR - REACH CCA |  |  |  | MX |  | NZ | PH | TH- TECI | TW | VN (Draft) |  |
| No |  |  |  | No |  | Yes | Yes | No | Yes | Yes |  |

## Dipropylene glycol monomethyl ether (34590-94-8)

| US | CA | AU | CN |  | EU |  | JP - ENCS | JP - ISHL |  | KR KECI - Annex 1 | KR KECI - Annex 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Yes | DSL | Yes | Yes |  | EIN |  | Yes | Yes |  | Yes | No |
| KR - REACH CCA |  |  |  | MX |  | NZ | PH | TH- TECI | TW | VN (Draft) |  |
| No |  |  |  | Yes |  | Yes | Yes | Yes | Yes | Yes |  |

## 2-Propanol, 1-propoxy- (1569-01-3)

| US | CA | AU | CN |  | EU |  | JP - ENCS | JP - ISHL |  | KR KECI - Annex 1 | KR KECI - Annex 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Yes | DSL | Yes | Yes |  | EIN |  | Yes | Yes |  | Yes | No |
| KR - REACH CCA |  |  |  | MX |  | NZ | PH | TH- TECI | TW | VN (Draft) |  |
| No |  |  |  | Yes |  | Yes | Yes | Yes | Yes | Yes |  |

## Poly(oxy-1,2-ethanediyl), .alpha.-undecyl-.omega.-hydroxy- (34398-01-1)

| US | CA | AU | CN |  | EU |  | JP - ENCS | JP - ISHL |  | KR KECI - Annex 1 | KR KECI - Annex 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Yes | DSL | Yes | Yes |  | No |  | No | No |  | Yes | No |
| KR - REACH CCA |  |  |  | MX |  | NZ | PH | TH- TECI | TW | VN (Draft) |  |
| No |  |  |  | No |  | Yes | Yes | No | Yes | Yes |  |

## Silicic acid, disodium salt (6834-92-0)

| US | CA | AU | CN |  | EU |  | JP - ENCS | JP - ISHL |  | KR KECI - Annex 1 | KR KECI - Annex 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Yes | DSL | Yes | Yes |  | EIN |  | Yes | Yes |  | Yes | No |
| KR - REACH CCA |  |  |  | MX |  | NZ | PH | TH- TECI | TW | VN (Draft) |  |
| No |  |  |  | Yes |  | Yes | Yes | Yes | Yes | Yes |  |

Section 16 - OTHER INFORMATION

## NFPA Ratings

Health: 1 Fire: 1 Instability: 0

Hazard Scale: 0 = Minimal 1 = Slight 2 = Moderate 3 = Serious 4 = Severe  Summary of Changes

Regulatory review and update. Key / Legend

ACGIH - American Conference of Governmental Industrial Hygienists; ADR - European Road Transport;

AU - Australia; BOD - Biochemical Oxygen Demand; C - Celsius; CA - Canada; CA/MA/MN/NJ/PA -

California/Massachusetts/Minnesota/New Jersey/Pennsylvania*; CAS - Chemical Abstracts Service;

CERCLA - Comprehensive Environmental Response, Compensation, and Liability Act; CFR - Code of

Federal Regulations (US); CLP - Classification, Labelling, and Packaging; CN - China; CPR - Controlled

Products Regulations; DFG - Deutsche Forschungsgemeinschaft; DOT - Department of Transportation; DSD - Dangerous Substance Directive; DSL - Domestic Substances List; EC – European Commission; EEC - European Economic Community; EIN - European Inventory of (Existing Commercial Chemical Substances);

EINECS - European Inventory of Existing Commercial Chemical Substances; ENCS - Japan Existing and

New Chemical Substance Inventory; EPA - Environmental Protection Agency; EU - European Union; F - Fahrenheit; F - Background (for Venezuela Biological Exposure Indices); IARC - International Agency for

Research on Cancer; IATA - International Air Transport Association; ICAO - International Civil Aviation

Organization; IDL - Ingredient Disclosure List; IDLH - Immediately Dangerous to Life and Health; IMDG -

International Maritime Dangerous Goods; ISHL - Japan Industrial Safety and Health Law; IUCLID - International Uniform Chemical Information Database; JP - Japan; Kow - Octanol/water partition coefficient; KR KECI Annex 1 - Korea Existing Chemicals Inventory (KECI) / Korea Existing Chemicals List (KECL); KR KECI Annex 2 - Korea Existing Chemicals Inventory (KECI) / Korea Existing Chemicals

List (KECL) , KR - Korea; LD50/LC50 - Lethal Dose/ Lethal Concentration; KR REACH CCA - Korea

Registration and Evaluation of Chemical Substances Chemical Control Act; LEL - Lower Explosive Limit;

LLV - Level Limit Value; LOLI - List Of LIsts™ - ChemADVISOR’s Regulatory Database; MAK - Maximum Concentration Value in the Workplace; MEL - Maximum Exposure Limits; MX – Mexico; Ne- Non-specific; NFPA - National Fire Protection Agency; NIOSH - National Institute for Occupational Safety and Health; NJTSR - New Jersey Trade Secret Registry; Nq - Non-quantitative; NSL – Non-Domestic Substance List (Canada); NTP - National Toxicology Program; NZ - New Zealand; OSHA - Occupational

Safety and Health Administration; PEL- Permissible Exposure Limit; PH - Philippines; RCRA - Resource

Conservation and Recovery Act; REACH- Registration, Evaluation, Authorisation, and restriction of

Chemicals; RID - European Rail Transport; SARA - Superfund Amendments and Reauthorization Act; Sc -

Semi-quantitative; STEL - Short-term Exposure Limit; TCCA – Korea Toxic Chemicals Control Act; TDG -

Transportation of Dangerous Goods; TH-TECI - Thailand - FDA Existing Chemicals Inventory (TECI);

TLV - Threshold Limit Value; TSCA - Toxic Substances Control Act; TW – Taiwan; TWA - Time

Weighted Average; UEL - Upper Explosive Limit; UN/NA - United Nations /North American; US - United States; VLE - Exposure Limit Value (Mexico); VN (Draft) - Vietnam (Draft); WHMIS - Workplace Hazardous Materials Information System (Canada).

Other Information Disclaimer:

User assumes all risks incident to the use of this product. To the best of our knowledge, the information contained herein is accurate. However, Safety-Kleen assumes no liability whatsoever for the accuracy or completeness of the information contained herein. No representations or warranties, either expressed or implied, of merchantability, fitness for a particular purpose or of any other nature are made hereunder with respect to the information or the product to which the information refers. The data contained on this sheet apply to the product as supplied to the user.