________________________________________________________________________

SECTION 1 - PRODUCT SOURCE AND IDENITY

SUPPLIER’S NAME AND IDENITY                   PRODUCT INFORMATION

Aegis Sealants                                                    NAME: Oxy Powder 007

4893 Klatte Road                                          Issue Date:  03/29/2025

Batavia, OH 45244                                             DESCRIPTION: Alkaline, Off-White Powder

________________________________________________________________________

EMERGENCY PHONE NO 1-513-315-7150  PRODUCT INFO (BUSINESS HOURS) 1-513-932-3396

________________________________________________________________________

SECTION 2     HAZARDS IDENTIFICATION

GHS-US Classification:  Skin:                           H315

Eye Damage:             2 H318

Aquatic Acute:          2 H401

Hazard Pictograms:  GHS-05

Signal Word:              Warning

Causes serious skin and eye damage. Do not breathe mist, spray, vapors.  Wash skin thoroughly after handling. Wear eye protection, face protection, protective clothing, protective gloves.  IF SWALLOWED: Rinse mouth, DO NOT induce vomiting.  IF ON SKIN rinse skin thoroughly, remove contaminated clothing. IF INHALED get to fresh air. IF IN EYES rinse thoroughly with water, remove contact lenses if present, rinse again.  Where symptoms exist call a poison center and/or a physician. Wash contaminated clothing.  Store in a secure location. Dispose of contents/container to comply with applicable regulations.

________________________________________________________________________

SECTION 3 –COMPOSITION/INFORMATION HAZARDOUS INGREDIENTS

Name                                 Product Identification    %        GHS-US Classification

Sodium Carbonate Peroxyhydrate       CAS 15630-89-4                   2-10         ACUTE TOX:; (dermal) H315,

Oral: 4 H302,  Eye Dam: 1 H318,

Aquatic Acute: 2 H401, Ox Sol: 3 H272.

Sodium Carbonate                               CAS  497-19-8                     40-70 %             Eye: irritant. 2A H319

Skin: irritant H315

This blend contains other proprietary non-DOT regulated surfactants and compounds.

__________________________________________________________________________________

HMIS  HEALTH   2                                  HMIS REACTIVITY  1   CONTACT 2

HMIS PERSONAL PROTECTION  D     HMIS FLAMMABILITY 0                      ______________________________________________________________________________________

EFFECTS/SYMPTOMS OF ROUTES OF ENTRY

Eyes: Causes severe eye damage

Skin:  Causes skin irritation. Prolonged exposure of dust may cause

allergic reaction to sensitive skin. (Wash often with soap and water).

Ingestion: Abdominal pain, bleeding of the gastrointestinal tract, nausea

Possible esophageal perforation

Inhalation:  Coughing, irritation of respiratory tract.

Page 1 of 4 pages

________________________________________________________________________

SECTION 4 – FIRST AID MEASURES

Eyes:  Flush from eyes with clean potable water.  See a physician if irritation persists.

Skin:  Wash from skin with soap and water.  Remove contaminated

clothing.  See a physician if irritation persists.

Ingestion: Do not  induce vomiting. Never give anything by mouth to an unconscious

person. Rinse mouth.  See a physician.

Inhalation: Get person to fresh air. If breathing problems

persist, administer oxygen, see a physician.

________________________________________________________________________

SECTION 5- FIRE-FIGHTING MEASURES

Fire: Extinguishing media:   Water, Carbon Dioxide.      Flammable: No

Firefighters:  Do not spread, do not use a heavy (high pressure) water stream.

Use protective equipment when entering area. Include respiratory equipment.     ______________________________________________________________________

SECTION 6 - ACCIDENTIAL RELEASE MEASURES

Spills:  For spills, sweep up any powder and absorb solutions onto inert absorbent

material such as bicarbonate or sweeping-compound and shovel into recovery

drums. Small spills may be rinsed away with water. Use goggles and dust

respirator and protect from getting onto skin and into eyes. Rinse from skin

and eyes. Avoid dust and fumes. Prevent entry into sewers and waterways.

Obey all state, local, and federal laws.

________________________________________________________________________

SECTION 7 – HANDLING AND STORAGE

Storage:  Store in closed container in a cool dry area out of the sun.  Avoid long direct exposure, and/or getting concentrated dust and liquid dilutions on skin, into

eyes, or on clothing.  Wash often when handling concentrate.

Incompatible Products: Acids, soft metals, strong alkaline material.

________________________________________________________________________     SECTION 8 - EXPOSURE CONTROLS AND PERSONAL PROTECTION

Protective Equipment; Use good ventilation, local exhaust, when handling dust.

Eyes:  Goggles or face shield.

Skin:  Chemical resistant gloves, and protective clothing such as a lab coat or apron.

Inhalation: Dust respirator where ventilation is insufficient.

Ingestion:  Avoid ingestion, use a dust mask and face shield where dust exists.

Page 2 of 4 pages

________________________________________________________________________

SECTION 9 – PHYSICAL AND CHEMICAL PROPERTIES

Appearance:         White to slight yellow Powder      Flash Point:              None

Odor:                    Varies: Odorless or lemon            Boiling Point:            N/A

Density:                0.958 ( 8.0 lbs/gallon)                  Vapor Pressure:          N/AV

Freezing Point:     None                                              Solubility in water    100 %

PH  Solution;        11.0-11.8

________________________________________________________________________

SECTION 10 - STABILITY AND REACTIVITY

Chemical Stability: Stable under normal conditions.    Flammability: N/A

Hazardous Polymerization: Will not occur.

Thermal Decomposition: Generates corrosive vapors.

Conditions to Avoid:  Avoid heat, sun, ignition sources. Avoid strong acids.

________________________________________________________________________

SECTION 11 - TOXICOLOGICAL INFORMATION

Oral Rat: LD50: 4090 mg/kg.  Inhalation Rat: 2300 mg/kg. Oral human: 714 mg/kg

Acute Toxicity: Not classified.  No known carcinogenic ingredients.

SECTION 12 - ECOLOGICAL INFORMATION

LC50 Fish 1:  300 mg/l       EC50 Daphnia 1: 265 mg/l       LC 50   Fish 2:750 mg/l

SECTION 13 – DISPOSAL CONSIDERATIONS

Waste Disposal Method: Sweep and place in waste disposal container. Neutralize

residue with water. Finish by spreading water on surface.

Dilute small spills with water and wash to sewer.

Dispose of in accordance with federal, state, and local law.

________________________________________________________________________

SECTION 14 – TRANSPORTATION INFORMATION

Transport Land: Not DOT regulated.                                                                                                        ________________________________________________________________________

SECTION 15 – REGULATORY INFORMATION

SARA 311/312 Hazard: No.

Immediate Hazard: Yes.

Delayed Hazard: No.

Pressure Hazard: Yes, avoid mixing with water inside a non-ventilated container.

Page 3 of 4 pages

_______________________________________________________________________

SECTION 16 – OTHER INFORMATION

Key Legend Information and Listing:

N/AV         Not Available

N/A –         Not Applicable

ACGIH –   American Conference of Governmental Industrial Hygienists

OSHA –    Occupational Safety and Health Administration

TLV –        Threshold Limit Value

TSCS –      Toxic Substance Control Act

CERCLA – Comprehensive Environmental Response, Compensation, and Liability Act

SARA –      Superfund Amendments and Reauthorization Act

HMIS –      Hazardous Materials Identification System—Class D-28 Material

WHMIS –   Workplace Hazardous Materials Information System

AIHA –      American Industrial Hygiene Association

The information contained herein is based on available data and is believed to be correct, however; no warranty is expressed or implied, regarding the accuracy of this data or the results to be obtained from the use thereof.

__________________________________

This section intentionally blank

Page 4 of 4 pages

_______________________________________________________________________